OPTICAL ROTATION MEASUREMENTS IN FOOD INDUSTRY PROCESSES

Daniel López(A,B), Patricia Mendoza-Sánchez(A,B), Antonio Torres(B), Gonzalo Ramos(A) ,

Reynaldo C. Pless(A) , Jorge Huerta-Ruelas(A,B)

(A) CICATA-IPN

José Siurob 10, Col. Alameda, Querétaro, 76040, México

(442) 2121111, 2121111 ext. 103, jhuerta@cicataqro.ipn.mx

(B)Dept. of Food Science & Technology, Oregon State University

100 Wiegand Hall, Corvallis, OR 97331-6602

Abstract: An optical system was developed using commercial low-cost optical components to monitor food processes by measuring changes in optical rotation (OR) of chiral compounds. The OR signal as a function of processing time and the sample temperature were collected and recorded using a computer data acquisition system. Sucrose (1 g/100 ml solution) and distilled water were used to calibrate the instrument. Sucrose solutions (0.01 to 30 g/100 ml solution) were used to determine its sensitivity, linearity and stability. The instrument was validated by monitoring the hydrolysis of sucrose. System has been tested during a fermentation process and sugar-protein interaction as a function of temperature.